Installation manuals

BC-2008NT

Single-door control systems

Sebury Technology Co.,Ltd

Installation manuals

BC-2008NT

Single-door control systems

- summarize

This system is divided into two parts: hardware and software, this manual just introduces installation, test for project installing, system maintaining.

BC-2008NT system is integrated with computer technology, the micro control technology, the smart card technology, the integration of machinery technology and so on with modernized intelligence access management using the non-contact induction smart card to take the status recognition; this system supports EM card, between the card and the controller communication using  Wiegand26 form to carry on for communications , the controller and the computer communication selects 485 main line methods, this system can control 99 gate areas totally , basically it can satisfy the user every of demand, and realize  difference management automation

features :

- Built-in pin keypad

- Built-in wiegand26 reader.

- Support door status monitors

- Support 5 kinds ways to open the door : card only, pin only, card/pin , card  & pin , external button

- Every controller can support 3000 cards, 3000 pins, and over 700 records.

- Provides 60 time zones, 32 time intervals

- Using the non-contact induction smart card to take the status recognition

- Supports EM cards

- Realize alarm linkage including force alarm, break-in alarm ,out-time alarm  for  pin keypad reader .

- Supports break-in alarm ,and out-time alarm for common reader;

- The alarm functions can be set  separately ;

- system can be used in a group ,can supports up to 99 doors .

- Indexes

| index | Power | Controller |
| --- | --- | --- |
| Voltage(V) | AC160 ～ AC250 | DC12～DC15 |
| currents(mA) | 20（statics） | 100±10% |
| Output vol(V) | DC12±5% | DC12±5% |
| Out-put current(A) | ≤3 | ≤2.5 |
| Net work distance(m) |  | ≤1200 |
| Temperature(℃) | -10～60 | -10～60 |
| Humidity (%) | 0～85 | 0～90 |
| Reading range(cm) |  | 3～8 |

- Appearance introduces

- Setting controller ID :

There are 99 identifications in this system, using 8421 BCD code, through move DIP switch, get separate identification code.

Note: than the identification code was set, must be re-power the controller.

Move to “ON”

1

2

3

4

5

6

7

8

For example

Tens digit

ON

8

4

2

1

8

4

2

1

Rest

ON

1

2

3

4

5

6

7

8

ID：05

ON

1

2

3

4

5

6

7

8

ID：97

- Hookup

Ports details

| number | colors | ports | Functions | remarks |
| --- | --- | --- | --- | --- |
| 10 | orange | RS485 B | Switch RS485 B | 485 communication |
| 9 | purple | RS485 A | Switch RS485 A |  |
| 8 | red | +12V | Power “ +” | Power input |
| 7 | black | GND | Power” –“ |  |
| 6 | blue | COM | Relay com | Relay output |
| 5 | white | NO | Relay N/O |  |
| 4 | green | NC | Relay N/C |  |
| 3 | brown | D-IN | Door status inspector | Door status |
| 2 | yellow | OPEN | Buzzer control | Low lever vol to open door |
| 1 | grey | NOT CONNECT | Not contact | Free ,not in use |

- Note:

- Please guarantee to get a correct code between 1 and 99;

- Please confirm no same codes between the controllers;

- If no communications between controllers and PC, please check the 485 communication   if wrong connected;

- Fail to open doors, check if the time set is valid; confirm the time zone and time terminal for this card is correct; please if your card is valid.

- Istallation: